Title of the work

Student name,

E-mail

### Abstract

Landslides falling into water bodies can generate destructive wave, which can be accounted as tsunamis. An experimental facility to study that phenomenon has been set up. It consists of a landslide generator releasing gravel at high-speed in a wave basin. A non-intrusive system has been designed ad-hoc to be able to measure the high velocity and the geometry of the landslide as well as the produced wave characteristics. The measurement system employs the treatment of images captured by a high-speed camera which records the launched granular material lighted by a laser sheet. A grid of laser sheets marks the basin water surface. The water has been filled by a small amount of kaolin to properly reflect the laser light at water surface. Thus, by filming with high definition cameras the perturbed water surface and successively processing the resulting images, it has been possible to measure the produced wave. The measurement framework employs a versatile camera calibration technique which allows accurate measurements in presence of: (1) high lens distortions; (2) pronounced non-parallelism condition between camera sensor and plane of measurement coincident with the laser sheet. The maximum resolution of the measurement tool is 0.01 mm while the maximum uncertainty due to systematic error has been estimated in 15 % for the worst scenario. This work reveals the suitability of image-based measuring systems in granular flows and free surface hydraulics experimental applications.

Keywords: camera calibration; granular flow measurement; landslide tsunami; video-image processing; wave profile measurement.

### Introduction

Computer vision has received a boost in the last 30 years mainly because the recent inexpensive and technically improved digital cameras are attractive for low cost measuring systems (Shapiro and Stockman, 2001). Image-based measuring systems are suitable for laboratories of hydraulics and have been recently used in several experimental practices (Yao and Wu 2005; Erikson and Hanson 2005; Iglesias et al. 2009; Wang et al. 2012).

A specific application of image-based measurements has been set-up in the Fluvial Morphodynamic I from GITS laboratories at the Technical University of Catalonia in order to study tsunamis generated by landslides (Bregoli et al. 2017). When a landslide falls into a water body a huge impulsive wave can be generated (Kamphuis and Bowering 1970). This wave is considered a tsunami for its great amplitude and length, its capacity of travelling long distances as well as its potential destructivity. The mentioned experimental facility has been set-up to define empirical relationships between landslide parameters and produced wave characteristics as tools useful for risk assessment. The system consists of a landslide generator releasing a granular mass of white gravel at high-speed (up to 7 m/s) in a wave basin of 4.10 × 2.44 m with a water depth hw ≥ 0.20 m. The landslide generator is formed by a sliding box, filled with the gravel, which runs down a high-slope flume (Fig. 1). When the box suddenly stops at the end of the flume, the flaps open and the granular material is launched into the wave basin (Fig. 2 and Fig. 3).

2 Measuring system

The images of the experiments are captured through an array of different video cameras. The fast landslide is recorded by a BASLER A504k high-speed (HS) industrial camera, able to shoot up to 1000 frames per second (FPS) and focused upon the exit of the landslide generator (Fig. 2, Fig. 3 and Fig. 4). The HS Camera has a fast monochrome digital CMOS sensor having resolution of 1280×1024 pixels and pixel size of 12x12 μm. The 8 GB dedicated RAM ensures the quick storage of up to 8 seconds of video clip at full resolution. This time is widely sufficient to record the entire landslide evolution. The HS Camera mounts the Nikon AF Nikkor 50mm f/1.4D lens. Besides its narrow focal length, the lens was selected thanks to its wide aperture: the high speed camera needs a large amount of light in order to capture readable images at high frequency and short exposure time.

3 Calibration techniques

The recorded video frames need a proper calibration to be translated into the spatial references of the measurements. Camera calibration in the context of computer vision is a technique extensively used in robotics and automation. This technique allows to define the parameters of transformation between the spatial references of the camera (images) and the spatial references of the measurement (world), through the optimization of a number of parameters (Tsai, 1987).

The parameters are divided in two subsets: intrinsic parameters (known camera internal geometric and optical characteristics such as the focal and the CMOS sensor characteristics); extrinsic parameters (3D position and orientation of the camera frame relative to a certain world system of coordinates). The optimization can be linear or nonlinear. Linear optimizations require less effort to be solved and are more stable than nonlinear ones. But, on the contrary of nonlinear optimization, linear optimization ignores lens distortions. In our experimental application, lens distortion is crucial and unknown because of the use of an inherently distorted commercial lens. Thus, the nonlinear pinhole method of Tsai (1987), including the optimization of lens distortion, is used.

An example of measured vs(t) and hs(t) is presented in Fig. 8. In researching hs, vs and ls a Lagrangian average on the entire granular mass passing through the control volume is calculated, considering total mass M and momentum p for unit width as follow:

4.2 Wave surface measurements

The water surface elevation is measured along the direction r for each time step fixed by the HD1 camera frame rate as Δt = 1/50 = 0.02s. The first frame when the contact of landslide with water is detected corresponds to the frame of initial time t = 0. In Fig. 10 it is possible to observe the vertical water surface displacement  measured from the frame reported in Fig. 9. Further extensive measurements are depicted in Bregoli et al. (2017). In Fig. 10 one observes that the measurement starts from r = 1m: the area of measurement was masked close to impact because of splash interferences.

Figure 1. Sketch of the measuring system.

Table 1. Maximum systematic errors and associated uncertainty in measurements

|  | HS Camera |  |  | HD1 Camera |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference axes | x | y | z | x | y | z |
| Max. error in trasl. / rot. | ± 1.90 ° | ± 1.27° | ± 5 mm | ± 0.57° | ± 0.38° | ± 5 mm |
| Max. uncertainty in x (%) | ± 0.38 | ± 2.16 | ± 0.39 | ± 0.11 | ± 0.65 | ± 0.39 |
| Max. uncertainty in y (%) | ± 11.00 | ± 3.62 | ± 15.02 | ± 5.22 | ± 2.90 | ± 15.02 |

### Conclusión

A system able to measure the main parameters of experiments of tsunamis generated by landslides have been presented. The measurement tool is based on computer vision and employs a versatile camera calibration technique based on the Tsai’s pinhole method (Tsai 1987). The calibration method is here improved to allow the correct measurement in presence of both lens and perspective high distortions. This last feature has been requested by the pronounced angle of view of the cameras been constrained by the experimental set-up features.

The system permitted to measure the rapid granular landslide form and velocity as well as the produced water displacement. Particularly the methodology allowed measuring the wave displacement at high density in space and high frequency in time, avoiding the classical point measurements given by probes and depicting wave series measurement, which is here not space dependent. For the specific experiments of tsunamis generated by landslides the space independent measurement is particularly beneficial because the point of maximum wave amplitude condition, corresponding with the beginning of wave propagation, forms at a certain distance from impact and it is strongly dependent on the landslide characteristics, thus varying for each run initial conditions (Bregoli et al. 2017).

### References

Pedro Karin Serrato Álvarez (2017) Geomorphological study and the river dynamics of the amazon river through a multitemporary analysis. Agustin Codazi. 10.13140/RG.2.2.23477.70880